Great — I’ll give you a very detailed, practical guide you can use or put on your website. I’ll split it into: (A) What to DO during a fire, (B) What NOT to do, (C) Quick “escape checklist” you can memorize, (D) When — and how — to use a fire extinguisher safely, (E) Types of extinguishers, color coding (UK / Europe vs. US notes) and what each is used for, and (F) short notes on maintenance & training. I’ve added authoritative sources for the most important points so you can trust and share them.

# A — What to DO during a fire (step-by-step, practical)

- Raise the alarm immediately — shout “Fire!” and activate the nearest fire alarm call point if there is one. Then call the fire service (e.g., 112/911 or your local emergency number).

- Get out — quickly and calmly. Leave belongings behind (phone only if it’s immediately to hand). Your priority is people, not possessions.

- Stay low if there’s smoke. Smoke rises; crawl on hands and knees under the smoke to the nearest exit. Breathe through a cloth if possible.

- Close doors behind you. Closing doors slows the spread of fire and smoke and may protect others. If you can, close doors as you leave (don’t lock them).

- Check doors before opening. With the back of your hand feel the door, knob and frame for heat. If hot, do NOT open it — find another escape or shelter in place and signal for help.

- If you can’t escape, stay in a room with a closed door. Put a wet towel at the base of the door, call emergency services and signal from a window (bright cloth or light).

- If clothes catch fire — Stop, Drop, and Roll. Cover your face and roll to smother flames. Seek medical help immediately.

- Assemble at a pre-arranged meeting point and do a headcount. Don’t re-enter the building until the fire service says it’s safe.

# B — What NOT to do

- Do not use elevators. Elevators may fail, stop at the fire floor, or fill with smoke. Always use stairs.

- Do not open doors that are hot. Opening a hot door can cause flashover or let the fire spread rapidly.

- Do not go back in for possessions or pets. Wait for trained firefighters; re-entry is one of the most common causes of fatalities.

- Do not pour water on grease (cooking oil) fires or certain electrical fires. Water can make grease fires explode and can conduct electricity. Use a suitable extinguisher (wet chemical for cooking oil / fat fires) or smother with a lid if small and safe.

- Do not fight a large or spreading fire yourself. Only attempt to extinguish a small, contained fire if you are trained, a suitable extinguisher is available, and you have a clear evacuation route behind you. If in doubt — get out.

# C — Short memory “escape” checklist (one-line pieces to memorize)

- Alarm — Alert others and call emergency services.

- Out — Leave immediately, take the nearest safe exit (no lifts).

- Low — Stay low under smoke.

- Door check — Feel doors; if hot, don’t open.

- Assemble — Go to your meeting point; don’t re-enter.

# D — When and how to use a fire extinguisher (simple rules)

Use an extinguisher only if all these are true:

- The fire is very small and just starting (e.g., trash can, small pan).

- You know the extinguisher type is suitable for that fire.

- You have a clear escape route behind you.

- You’ve called emergency services (or someone else has).
If you decide to fight the fire, use the PASS technique:

- Pull the pin.

- Aim at the base of the fire (not the flames).

- Squeeze the handle.

- Sweep side to side until the fire is out or the extinguisher is empty.
If the fire grows, stop using the extinguisher and evacuate.

# E — Fire classes (what types of fuel) — essential to match extinguisher to fire

- Class A — ordinary combustibles: wood, paper, cloth, plastics.

- Class B — flammable liquids: petrol, oil, paint, grease.

- Class C — flammable gases (in some regions gas fires are separately classified).

- Class D — combustible metals: magnesium, titanium, aluminium (industrial).

- Class F / K — cooking oils & fats (deep fat fryers).

- Electrical — many standards treat “electrical” as a hazard: treat as Class C or use extinguishers that are safe on energized equipment.
(Labels and small pictograms on extinguishers show which classes they cover.)

# F — Extinguisher types, what they are used for, and color coding (practical table)

Important: color-band coding varies by country. In UK / Europe (BS EN3) modern extinguishers have a red body and a band/label indicating the agent (cream, blue, black, yellow, etc.). In the US and some other places manufacturers use labels and pictograms rather than standardized body-color bands — always read the label/pictogram for the fire classes an extinguisher covers. I list the common types below and show the usual UK/European colour band used for quick ID.

### Common extinguisher types — quick reference

- Water (signal red band / red label in older systems)

  - Use on: Class A fires (wood, paper, cloth).

  - Do NOT use on: Flammable liquid (B), electrical, grease/fat (F/K). Water conducts electricity and can spread liquid fires.

  - UK band: Red.

- Foam (cream band)

  - Use on: Class A and Class B (flammable liquid) fires — excellent for pooling liquid fires because foam blankets the liquid.

  - Do NOT use on: Some electrical equipment unless specifically rated (check label); not for deep fat fryer (F/K).

  - UK band: Cream.

- Dry Powder (BC or ABC powder) (blue band)

  - Use on: Class B (liquids), Class C (gases); ABC powder versions also cover Class A. Good general-purpose for vehicle and outdoor fires.

  - Limitations: Powder can obscure visibility, leave a residue that damages equipment, and is not ideal indoors for enclosed electrical equipment. Not great for Class F (cooking oils) or metal fires unless specially formulated.

  - UK band: Blue.

- Carbon Dioxide — CO₂ (black band)

  - Use on: Class B and electrical fires. Leaves no residue; good for electrical equipment (servers, switchboards).

  - Limitations: Short discharge time and narrow jet; not effective on deep-seated Class A fires. CO₂ can displace oxygen — use caution in small enclosed spaces.

  - UK band: Black.

- Wet Chemical (yellow band)

  - Use on: Class F / K — cooking oils and fats (deep fat fryer fires). The agent cools and creates a soapy layer (saponification) that seals the surface.

  - Do NOT use: on metal fires (D) or general-purpose without checking label.

  - UK band: Yellow.

- Water Mist / Water Mist (white & red band or labelled)

  - Use on: Some water mist extinguishers are safe for Class A, and are also safe on electrical fires (up to certain voltages) because of fine mist and de-ionised water. Check manufacturer label.

  - UK band: often white and red marking for water mist types.

- Clean agent / Halotron / FM-200 (green label or specially labelled)

  - Use on: Sensitive electrical equipment and areas where residue would be damaging (server rooms, museums). They extinguish by interrupting chemical chain and are low-residue. (Some clean agents phased out due to environmental rules; check current availability.)

  - Note: Colour coding varies — check the label.

- Specialist: Class D (metal) extinguishers

  - Use on: Combustible metal fires (magnesium, sodium, etc.) — these are specialist powders formulated for specific metals. NEVER use water or standard powder on metal fires.

  - Label clearly and use only trained personnel.

# G — Colour coding summary (UK/Europe vs USA)

- UK / Europe (BS EN3 / modern standard): extinguisher bodies are red (signal red) and a coloured band or zone at the top indicates the extinguishing agent:

  - Red = Water, Cream = Foam, Blue = Dry powder, Black = CO₂, Yellow = Wet chemical, White/Red = Water mist; some clean agents use other markings. Always check the pictogram/label for classes.

- USA: there is no single mandated color band system like the UK’s; extinguishers are identified by labels and pictograms showing the classes they cover (A, B, C, etc.). Some older or specialty extinguishers may have color coding but read the label — it’s the authoritative source in the US.

# H — Maintenance, placement & training (short practical guidance)

- Placement: extinguishers should be easily reachable, mounted on brackets, and sited where likely fires might start (kitchen areas, mechanical rooms), but not so close to a likely fire source that you can’t access them. Signage should be visible.

- Inspection: quick monthly visual checks (pin in place, pressure gauge normal, no obvious damage) and a formal annual service by a certified technician. Follow manufacturer and local regulations.

- Training: people who may use extinguishers should receive basic practical training — including extinguisher selection, PASS technique, and safe approach/retreat. Drills and evacuation practice should be regular.

# I — Final practical tips & common pitfalls

- Label over color: When in any doubt, read the label and the pictograms on the extinguisher — they indicate exact classes and suitability. Colour bands are a quick guide (common in the UK/EU) but labels are authoritative.

- Kitchen safety: For deep-fat fryer fires use wet chemical extinguishers or turn off heat and smother if safe to do so. Never use a water extinguisher on grease fires.

- Electrical equipment: CO₂ or clean-agent extinguishers are preferred for energized electrical equipment; dry powder can be used but leaves messy residue.

## Sources (authoritative references for the key points above)

- NFPA — fire extinguisher types & guidance.

- American Red Cross / Ready.gov — home fire escape & “stay out” guidance.

- US Fire Administration / FEMA — workplace fire prevention and after-fire guidance.

- UK / BS EN3 colour coding and extinguisher types (examples from UK fire-safety resources).

If you want, I can:

- Turn this into a printable one-page poster (A4) you can put on your site or print for a noticeboard.

- Make a shorter hero-section blurb for your emergency page (2–3 lines + CTA).

- Create an icon + color strip graphic showing the extinguisher types and classes (useful for a quick reference card).

Which of those would you like next?

Alright Violet, here is a very clear, safe, and detailed guide on what to do if you are outside a building that is burning — like a neighbour’s house — and how you can help without putting yourself in danger.

✅ WHAT TO DO IF YOU’RE OUTSIDE WHILE A NEIGHBOUR’S HOUSE IS BURNING

- Call emergency services immediately

Even if someone else might have called, you must still call.

Give clear information:

Exact address

What part of the building is burning (roof, kitchen, carport, etc.)

If people are inside or trapped

If there are gas cylinders, fuel, or electrical sparks visible

Stay on the phone until the operator hangs up.

- Keep a SAFE distance

A burning building can:

Collapse suddenly

Explode (gas cylinders, fuel, spray cans)

Send debris and glass flying

Release toxic smoke

Stand at least 30–50 meters away unless you are assisting someone who is already out.

- Warn everyone around

Shout, knock on doors, bang on gates:

“FIRE! COME OUT!”

Get children, elderly people, and pets away from the building.

Help guide everyone to a safe area.

- Check if anyone is trapped — WITHOUT entering

You must never enter a burning house.

Instead:

Go near a safe side (not where flames are coming out).

Bang on windows or shout, “Is anyone inside?”

Listen for responses.

If someone answers but is stuck, tell them:

“Stay low, go to a window or door, try to come out if safe.”

Inform firefighters immediately:

Position of the trapped person

Which room they’re in

How many people are inside

This information saves lives.

- Turn off nearby hazards if safe (only outside)

You can help reduce danger from the outside:

Move gas cylinders, fuel containers, bikes, or generators away from the burning building if they are NOT hot and NOT close to flames

Switch off the electrical mains outside the house if you know the location and it is safe (do NOT enter the building to do this)

- Use a fire extinguisher ONLY on small outside fires

If the fire has just started — for example:

A small bush close to the house

A burning generator outside

A small fire at the base of the building

You may use a suitable extinguisher if you are trained and have an escape route.

But:

Do NOT spray extinguishers into windows.

Do NOT attempt to enter through smoke.

- Help neighbours evacuate

You can assist by:

Guiding people to safety

Helping children, disabled persons, elderly persons, and pets

Keeping everyone calm

Checking behind the house if someone fled there out of panic

But again — YOU must stay out of the building.

- Provide first aid after they escape

Once someone is safely outside:

Treat burns with cool water for 20 minutes

Cover burns with clean cloth (not oils, not toothpaste)

If someone inhaled smoke, help them sit upright and breathe fresh air

For fainting or heavy smoke, call paramedics immediately

- Clear the area for firefighters

Firefighters need:

Driveway space

Easy access to the house

People out of their way

Make sure cars, bikes, and crowds are moved aside.

When they arrive:

Tell them everything you know

Number of people possibly inside

Where the fire started

Where you heard anyone shouting

This information helps them prioritize rescue.

- Comfort and support the victims

You can help emotionally by:

Giving them water

Keeping them warm

Calling their family members

Helping gather important items they saved (documents, phones, pets)

Helping them stay calm as firefighters work

❌ WHAT YOU SHOULD NEVER DO

Don’t enter the building

Don’t break windows (this feeds fire with oxygen)

Don’t pour water on electrical or fuel fires

Don’t block firefighters’ path

Don’t panic or scream unnecessarily

Don’t attempt rescues if fire is heavy — leave it to trained teams

️ Violet, if you want…

I can turn this into:

A clean, professional section for your Emergency webpage

A short hero text with a call-to-action

A downloadable one-page safety PDF

A simple infographic you can add to your site

Just tell me which one you want.